Lab 10

To Design and Implement Multiplexer & Demultiplexer

Tasks

- Construct a logic circuit for 8 to 1 multiplexer with the help of truth table. Also write the Boolean expression for output(s). Simulate your circuit to verify the outputs.

8 to 1 Mux

- Block Diagram

D0

D1

D2

Q

D3

8 to 1

MUX

D4

D5

D6

D7

S0

S1

S2

- Truth Table

| S2 | S1 | S1 | Q |
| --- | --- | --- | --- |
| 0 | 0 | 0 | D0 |
| 0 | 0 | 1 | D1 |
| 0 | 1 | 0 | D2 |
| 0 | 1 | 1 | D3 |
| 1 | 0 | 0 | D4 |
| 1 | 0 | 1 | D5 |
| 1 | 1 | 0 | D6 |
| 1 | 1 | 1 | D7 |

- Boolean Expression

Q = S2’.S1’.S0’.D0 + S2’.S1’.S0.D1 + S2’.S1.S0’.D2 + S2’.S1.S0.D3 + S2.S1’.S0’.D4 + S2.S1’.S0.D5 + S2.S1.S0’.D6 + S2.S1.S0.D7

- Logic Diagram (from logicaly or hand drawn)

- Software Simulation

- Design a logic circuit for 1-to-4-line Demultiplexer. Also write the Boolean expression for output(s). Simulate your circuit to verify the outputs.

- Block Diagram

D0

D1

1 to 4

DEMUX

D3

D2

Q

S1

S0

- Truth Table

| S1 | S0 | Q |
| --- | --- | --- |
| 0 | 0 | D0 |
| 0 | 1 | D1 |
| 1 | 0 | D2 |
| 1 | 1 | D3 |

- Boolean Expression

D0 = S1’. S0’. Q

D1 = S1’. S0. Q

D2 = S1. S0’. Q

D3 = S1. S0. Q

- Logic Diagram

- Software Simulation

- Design a circuit for 4 to 1 Multiplexer using 2 to 1 Multiplexer(s).  You can take help from google or the link below. Just ignore the coding language discussed in the link.

https://bravelearn.com/design-of-4x2-multiplexer-using-2x1-mux-in-verilog/

- Block Diagram

S0

S1

2 x 1 MUX

Q

2 x 1 MUX

S3

S4

2 x 1 MUX

S0

S1

- Truth Table

| S1 | S0 | Q |
| --- | --- | --- |
| 0 | 0 | D0 |
| 0 | 1 | D1 |
| 1 | 0 | D2 |
| 1 | 1 | D3 |

- Logic Circuit (on the basis of 2 to 1 Muxes used/follow the block diagram to draw this circuit)

You need to connect three 2 x1 Multiplexers in order to make one 4x1 Multiplexer.