Lab 6

To Demonstrate the Working of Binary Subtractor

Note: You may draw all the logic diagrams with hand and paste the pictures here or on logicly software with your name, roll number & section mentioned in your workspace. Make sure that all of your connections are clearly visible and distinguishable.

Tasks

- Construct a logic circuit for half and full subtractor with the help of truth table/Boolean expression. Also write the Boolean expression for output(s).

Half Subtractor

- Truth Table

| A | B | Difference | Borrow |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |

- Boolean Expression (Simplified)

Difference:

D = A X-OR B = A⊕B

Borrow:

B = NOT A AND B = A’.B

- Logic Diagram

- Software Simulation (Show here your results for each combination that gives a high output)

Full Subtractor

- Truth Table

| A | B | Borrow in | Difference | Borrow out |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

- Boolean Expression (Simplified)

Difference:

D = (A X-OR B) XOR BIN = (A ⊕ B) ⊕ BIN

Borrow:

BOUT = (NOT A AND B) OR {NOT (A X-OR B)} BIN=A’.B +{(A ⊕ B)}’BIN

- Logic Diagram

- Software Simulation (Show here your results for each combination that gives a high output)

- A full subtractor can be implemented using 2-half subtractors. Demonstrate the logic diagram for the said circuit. Simulate your circuit for the verification of results.

- Logic Diagram of Full Subtractor using 2-Half Subtractor

- Software Simulation (Show here your results for each combination that gives a high output)